Item no.1 Part No. CTB484820S1. (RU001A-LCP-01 & RU001B-LCP-01))

Junction box is made in stainless steel 316L. Ex II 2GD Ex deb IIC T Gb Ex tb IIIC T°C Db, IP66.

Ambient temperature -20 +55°C. Lid fixed with hinges and screws.

1 removable gland plate: bottom side. External dimension 480x480x205mm.

1xSS mounting plate.

1x SS breather with Lock Nut.

5x 24V DC Indicating lamp. (Please confirm the color of the each lamp)

3x Green Pushbutton with 1NO contact block.

3 x Red Push button with 1NC contact block.

2x 10A 2 POSITION (L-A/R)Selector switch with external handle.

2x 10A 3 POSITION (H-O-A)Selector switch with external handle.

TB-1: 12x2.5sq.mm terminal.(WDU2.5)

TB-2: 12x2.5sq.mm terminal.(WDU2.5)

TB-3: 12x2.5sq.mm terminal.(WDU2.5)

TB-4: 12x2.5sq.mm terminal.(WDU2.5)

4X M25x1.5 Brass Ni-Plated Cable Gland with Lock-Nut.

1X M40x1.5 Brass Ni-Plated Cable Gland with Lock-Nut.

Cable entry details.(Please confirm the cable entry)

1x41mm hole suitable for M40 cable entry at the bottom.

4x26mm hole suitable for M26 cable entry at the bottom.

https://pim.cortemgroup.com/uploads/pim/originals/13-ctb-en.pdf

Unit Price in Euro: 2,500.00.

Qty: 2 Nos.

Total in Euro: 5,000.00.

Item no.2:- PART NO.: CTB453820S1. (JBD-A-RU0001A-01 & JBD-A-RU0001B-01)

Junction box made in stainless steel 316L. Ex II 2GD Ex eb IIC T Gb Ex tb IIIC T°C Db, IP66.

Ambient temperature -20 +55°C. Lid fixed with hinges and screws.

1 removable gland plate: bottom side. External dimension 450x380x205mm.

1 x SS Mounting Plate.

1 X M8 Earthing stud.

1x M20x1.5 SS SS with Locknut.

TB-1: 24x2.5sq.mm terminal.(WDU2.5)

TB-2: 12x2.5sq.mm terminal.(WDU2.5)

2x M20x1.5 Brass Ni-Plated stopping plug with lock-nut.

Cable entry details. (Please confirm the cable entry)

1x26mm hole suitable for M25 cable entry at the bottom.

15x221mm hole suitable for M20 cable entry at the bottom.

https://pim.cortemgroup.com/uploads/pim/originals/14-ctb_en.pdf

Unit Price in EURO: 1,250.00

Qty: 2 Nos.

Total Price in EURO: 2,500.00

Item no.3:- PART NO.: CTB303020S1.(JBD-A-RU0001A-03 & JBD-A-RU0001B-03)

Junction box made in stainless steel 316L. Ex II 2GD Ex eb IIC T Gb Ex tb IIIC T°C Db, IP66.

Ambient temperature -20 +55°C. Lid fixed with hinges and screws.

1 removable gland plate: bottom side. External dimension 306x306x205mm.

1 x SS Mounting Plate.

1 X M8 Earthing stud.

1x M20x1.5 SS SS with Locknut.

TB-1: 30x2.5sq.mm terminal.(WDU2.5)

2x M20x1.5 Brass Ni-Plated stopping plug with lock-nut.

Cable entry details. (Please confirm the cable entry)

1x26mm hole suitable for M25 cable entry at the bottom.

10x221mm hole suitable for M20 cable entry at the bottom.

https://pim.cortemgroup.com/uploads/pim/originals/14-ctb_en.pdf

Unit Price in EURO: 800.00

Qty: 2 Nos.

Total Price in EURO: 1,600.00

Item no.4:- PART NO.: CTB262620S1.(JBD-A-RU0001A-02 & D-A-RU0001B-02)

Junction box made in stainless steel 316L. Ex II 2GD Ex eb IIC T Gb Ex tb IIIC T°C Db, IP66.

Ambient temperature -20 +55°C. Lid fixed with hinges and screws.

1 removable gland plate: bottom side. External dimension 260x260x205mm.

1 x SS Mounting Plate.

1 X M8 Earthing stud.

1x M20x1.5 SS SS with Locknut.

TB-1: 16x2.5sq.mm terminal.(WDU2.5)

2x M20x1.5 Brass Ni-Plated stopping plug with lock-nut.

Cable entry details. (Please confirm the cable entry)

1x26mm hole suitable for M25 cable entry at the bottom.

12x221mm hole suitable for M20 cable entry at the bottom.

https://pim.cortemgroup.com/uploads/pim/originals/14-ctb_en.pdf

Unit Price in EURO: 600.00

Qty: 2 Nos.

Total Price in EURO: 1,200.00

Item no.5:- PART NO.: CTB262620S1.(JBD-A-RU0001A-04 & JBD-A-RU0001B-04)

Junction box made in stainless steel 316L. Ex II 2GD Ex eb IIC T Gb Ex tb IIIC T°C Db, IP66.

Ambient temperature -20 +55°C. Lid fixed with hinges and screws.

1 removable gland plate: bottom side. External dimension 260x260x205mm.

1 x SS Mounting Plate.

1 X M8 Earthing stud.

1x M20x1.5 SS SS with Locknut.

TB-1: 16x2.5sq.mm terminal.(WDU2.5)

2x M20x1.5 Brass Ni-Plated stopping plug with lock-nut.

Cable entry details. (Please confirm the cable entry)

1x26mm hole suitable for M25 cable entry at the bottom.

12x21mm hole suitable for M20 cable entry at the bottom.

https://pim.cortemgroup.com/uploads/pim/originals/14-ctb_en.pdf

Unit Price in EURO: 600.00

Qty: 2 Nos.

Total Price in EURO: 1,200.00

Item no.6 Part No. CTB221513S1.(TB-HTR-RU0001A & TB-HTR-RU0001B)

Junction box made in stainless steel 316L. Ex II 2GD Ex- deb IIC T Gb Ex tb IIIC T°C Db, IP66.

Ambient temperature -20 +55°C. Lid fixed with hinges and screws.

1 removable gland plate: bottom side. External dimension 229x152x130mm.

1 x SS mounting plate

1x M20x1.5 SS SS with Locknut.

2xGreen Pushbutton with 1NO contact block.

2x M20x1.5 BNP plug with lock-nut.

4x2.5sq.mm terminal (WDU2.5)

Cable entry:

3-21mm hole suitable for M20 at the bottom side.

https://pim.cortemgroup.com/uploads/pim/originals/13-ctb-en.pdf

Unit Price in Euro: 500.00.

Qty: 2 Nos.

Total in Euro: 1,000.00.

Terms & Conditions:

1.            Payment: 100% along with the Purchase Order

2.             The prices quoted are for Ex-works Jafza, UAE, It doesn't include VAT + Customs clearance + Documentation + Delivery.

3.            Delivery Time: The estimated delivery time will be 4-5 working weeks from that date of PO and approved GA drawing

4.             Our Tax Registration No. (TRN) 100380564300003.

5.             Offer Validity: 30 days

6.             Please note that if the order value is less than EUR 300, then a surcharge of EUR 50 is added to the original amount.

7.             70 extra for COO for attested.

8.             If inspection is applicable then TPI will be under your scope and there will 500 euro service charge per day.

Should you have any queries, please feel free to contact us.

Thanks!

Badruddin M. Khan

Sales Engineer

e-mail : B.Khan@cortemgroup.com

|  | Cortem GULF FZCO RA08HC2, RA08HC3 & RA08HC4 near Round#8, Jebel Ali Free Zone P.O.Box: 17123, Dubai United Arab Emirates Phone: +971 4806 2000 Fax: +971 4881 7609 Mobile: +971 56 504 4037. Web : www.cortemgroup.com |
| --- | --- |